NOR Cell

- The NOR cell implements the comparison between the complementary stored bit, D (and D), and the complementary search data on the complementary searchline, SL (and SL), using four comparison transistors, M1 through M4, which are all typically minimum-size to maintain high cell density

- The NOR nature of this cell becomes clear when multiple cells are connected in parallel to form a CAM word by shorting the ML of each cell to the ML of adjacent cells.

- There is a match condition on a given ML only if every individual cell in the word has a match.

NAND Cell

- The NAND cell implements the comparison between the stored bit, D, and corresponding search data on the corresponding searchlines, (SL, SL), using the three comparison transistors M1, MD, and MD, which are all typically minimum-size to maintain high cell density.

- The NAND nature of this cell becomes clear when multiple NAND cells are serially connected.

- It provides a full rail voltage at the gates of all comparison transistors. It provides a reduced logic “1” voltage at node B, which can reach only VDD - VTn when the searchlines are driven to VDD.

Cell Variant of NOR

- The NOR cell variant [Fig. a] uses only 9-transistors compared to the previous 10-T NOR cell.

- Although, this 9-T NOR cell has only a single transistor in the pulldown path, the node B can only rise to VDD - VTn so usually the 10-T NOR cell is preferred.

Cell Variant of NAND

- The 10-T NAND cell [Fig. b] is a variant of the previous 9-T NAND cell.

- Since this NAND cell doubles the number of transistors in series, the 9-T NAND is usually preferred.

NOR Matchline

NAND Matchline